Article

Evaluation of an ELIME assay for Hepatitis A Virus detection in drinking and vegetable processing water

Cristine D’Agostino1,2, Rocco Cancelliere1, Antonio Ceccarelli1,3, Danila Moscone1, Loredana Cozzi4, Giuseppina La Rosa5, Elisabetta Suffredini4 and Laura Micheli1,*

Citation: To be added by editorial staff during production. Academic Editor: Firstname Lastname Received: date Revised: date Accepted: date Published: date Copyright: © 2023 by the authors. Submitted for possible open access publication under the terms and conditions of the Creative Commons Attribution (CC BY) license (https://creativecommons.org/licenses/by/4.0/).

1  University of Rome Tor Vergata, via della Ricerca Scientifica 1, 00173, Rome, Italy; laura.micheli@uniroma2.it, rocco.cancelliere@uniroma2.it, antonio.ceccarelli@sanasrl.it, danila.moscone@uniroma2.it

2  University of Rome La Sapienza, piazza Aldo Moro 1, 00100 Rome, Italy; cristine.dagostino@uniroma1.it

3  Laboratorio di analisi agroalimentare Sana srl – Corso Lazio 17 031000, Frosinone, Italy; sanasrl.it3576@sanasrl.it, antonio.ceccarelli@sanasrl.it

4	Istituto Superiore di Sanità, Department of Food Safety, Nutrition and Veterinary public Health, v.le Regina Elena 299, 00161 Rome, Italy; elisabetta.suffredini@iss.it, loredana.cozzi@iss.it

5 	Istituto Superiore di Sanità, National Center for Water Safety (CeNSia), v.le Regina Elena 299, 00161 Rome, Italy; giuseppina.larosa@iss.it

*	Correspondence: laura.micheli@uniroma2.it

Abstract: Globally, waterborne viral infections pose a significant threat to public health. While current European Union regulations stipulate that drinking water must be devoid of harmful pathogens, they do not specifically address the presence of enteric viruses in water utilized for irrigation or food production. Traditional virus detection methods rely on molecular biology assays, which necessitate specialized personnel and laboratory facilities. Here, we describe an electrochemical sandwich enzyme-linked immunomagnetic assay (ELIME) for the detection of Hepatitis A Virus (HAV) in water matrices. This method employs screen-printed electrodes as the sensing platform and utilizes commercially available pre-activated magnetic beads to provide a robust foundation for the immunological reaction. The ELIME assay demonstrated exceptional analytical performance, achieving a detection limit of 0.5 genomic copies per milliliter (g.c./mL) and exhibiting good reproducibility with a relative standard deviation (RSD) of 7% in HAV-spiked drinking and processing water samples. Compared to the real-time RT-qPCR method described in ISO 15216-1, the ELIME assay demonstrated higher sensitivity, although the overall linearity of the method was moderate. These analytical attributes highlight the potential of the ELIME assay as a rapid and viable alternative for HAV detection in water used for agriculture and food processing.

Keywords: HAV; ELIME; screen-printed electrodes; drinking water; vegetable processing water.

1. Introduction

Hepatitis A virus (HAV), a non-enveloped, single-stranded positive-sense RNA virus within the Picornaviridae family, is the causative agent of acute viral hepatitis and is responsible for numerous cases and outbreaks globally [1,2]. HAV is primarily spread via the faecal-oral route, either through direct contact with infected individuals or through the ingestion of contaminated food (such as bivalve shellfish and soft fruits) or water [3,4]. The prevalence of food- and waterborne viral diseases represents a substantial public health challenge worldwide. A comprehensive review of HAV prevalence across various water matrices—including untreated and treated wastewater, surface water, groundwater, and drinking water—reported an overall prevalence of 16.7%, with the highest prevalence in untreated wastewater (31.4%) and the lowest in drinking water (0.3%) [5].

Currently, European Union legislation on water intended for human consumption (EU Dir. 2020/2184) depends on faecal indicator bacteria (FIB) for microbiological safety, which is more straightforward to measure compared to the full range of pathogens that pose health risks [6]. However, FIB does not adequately indicate the presence of pathogenic viruses. Similarly, regulations on microbial contamination in food products and hygiene in primary production (Reg. EC 2073/2005, 853/2004, 852/2004, and subsequent amendments) mandate the use of water deemed 'suitable for human consumption' or 'clean water,' yet lack specific microbiological criteria for enteric viruses in waters used for food production or irrigation [7–9]. Public health management strategies for hepatitis A emphasize pre-exposure vaccination for high-risk groups, education on good personal hygiene, particularly hand hygiene, and the provision of adequate sanitation and housing. Countries with high-income levels and robust sanitary conditions typically exhibit low infection rates. Understanding the environmental sources of HAV is critical for controlling the disease. Studies have identified HAV in foods, treated and untreated wastewater, and various aquatic environments [10–12].

The gold standard for the quantitative detection of HAV in food and bottled water is reverse transcription-quantitative real-time PCR (RT-qPCR), supported by an internationally validated method (ISO 15216-1:2017) [13,14]. However, while RT-qPCR and the more recent reverse transcription digital PCR (RT-dPCR) – which offers greater tolerance to inhibitors in food and water samples – are effective, these techniques have significant limitations [15]. These molecular assays, indeed, are labour-intensive and time-consuming as they are associated to ad hoc sample preparation procedures, require well-trained personnel with expertise in molecular biology, and necessitate specialized laboratory environments to minimize cross-contaminations and false-positive results, hence restricting their use to well-equipped facilities [16–19]. To address these limitations and the extended time required for these analyses, alternative methods have been developed, such as Enzyme-Linked Immunosorbent Assay (ELISA), which detect specific antigens through targeted antibodies [20–22]. ELISA tests are generally more cost-effective than PCR, making them suitable for high-throughput screening and allowing for rapid testing of large sample numbers [15,23]. Recent advancements have utilized magnetic beads modified with a secondary antibody to support the immunological reaction typical of ELISA, enhancing the method's efficiency even in complex matrices [24]. Compared to traditional ELISA, the ELIME (Enzyme-Linked Immunosorbent Magnetoelectrochemical) method offers a larger surface-to-volume ratio due to magnetic particles and improved sensitivity of electrochemical detection over spectrophotometric analysis [25–27].

This study highlights the advantages of ELIME over PCR and ELISA, demonstrating the sensitivity and reliability of the electrochemical sandwich ELIME technique, which employs commercially preactivated magnetic beads for HAV detection in processing water samples [28]. Our findings reveal significant benefits of this method compared to the real-time RT-qPCR protocol outlined in ISO 15216-1 [29], presenting promising results in analytical performance comparisons.

2. Materials and Methods

2.1. Reagents

Unless otherwise specified, chemical reagents were obtained from Merck® (Italy). Goat Anti-Mouse IgG magnetic beads were purchased from New England BioLabs® (UK) and stored at +4 °C. Anti-Mouse IgG is covalently coupled to a 1 μm nanoporous paramagnetic microparticle; 1 mL suspension in PBS Buffer (pH 7.4), 0.05% Tween 20, 0.1% BSA and 0.05% NaN3 contains 3.65×1010 particles. Primary Monoclonal Hepatitis A Virus Antibody (MAbI) clone M5112922 used in this study was provided by Fitzgerald® (USA), 1000 μg/mL in 10 mM PBS, pH 7.2, with 0.1% NaN3, whereas Primary Monoclonal Hepatitis A Virus Antibody labelled with Alkaline Phosphatase (MAbI-AP) clone 7E7 was supplied by Pantec® (Italy) in 10 mM PBS, pH 7.2, with 0.1% NaN3. Both Ab were stored at -20°C. The viral Standard for Hepatitis A virus was purchased from Mediagnost® (Germany). Information provided by the supplier, concerning the amount of HAV virus in the sample, indicated that the suspension contained 4.8×106 genome copies (g.c.)/mL (measured by real-time RT-qPCR according to ISO 15216-1:2017), and 100 International Unit (UI)/mL. Hepatitis A virus strain HM175/18f (American Type Culture Collection, ATCC VR-1402), used for spiking drinking water samples, was provided by the Istituto Superiore di Sanità (ISS) at concentration of 7.5×108 g.c./mL (measured by real-time RT-qPCR). All stocks of HAV were stored in aliquots at -20 °C or below until use. The buffers used for the experiments were the following: Solution A (washing magnetic beads solution): 50 mM sodium phosphate buffer (PB) pH 7.4 containing 100 mM KCl and 0.05% (v/v) of Tween 20; Solution B (store magnetic beads solution): 50 mM PB buffer pH 7.4 containing 0.1% BSA and 0.02% NaN3; Solution C (extraction solution): 100 mM Tris, 50 mM Gly and 0.1% of Beef Extract pH 9.5 (TGBE buffer).

2.2. Materials

2.2.1. Screen-printed electrodes (SPEs)

Screen-printed electrodes were produced by a screen printer DEK 245 (DEK, Weymouth, UK) on polyvinyl chloride (PVC) strips with eight printed sensors each. The inks were from Acheson (Italy). The SPE includes three electrodes: graphite for working and counter electrodes and silver/silver chloride (Ag/AgCl) for the reference electrode. The diameter of the working electrode was 0.3 cm, with an apparent geometric area of 0.07 cm2 [30,31].

2.2.2. Filters

Zeta Plus™ 1MDS discs (CUNO, 3M Company, USA), 47 mm, 0.45 μm pore size, were used in this study. Filters are characterized by an electropositive, charge-modified, cellulose medium double open-ended (flat nitrile gasket) configuration. The filters use electrokinetic adsorption to capture and concentrate waterborne viruses.

2.2.3. Instrumentations

Electrochemical measurements were performed at room temperature, using a computer-controlled system, AUTOLAB model GPSTAT-12 with GPES software (Ecochemie, Utrecht, The Netherlands). Differential pulse voltammetry (DPV) measurements were carried out in the potential range from 0-600 mV with a pulse width of 50 ms, a pulse amplitude of 70 mV and a scan speed of 100 mV/s.

2.3. Methods

2.3.1. ELIME assay

In this work, an Enzyme-Linked Immuno-Magnetic Electrochemical assay (ELIME), with a sandwich format, was developed. Goat Anti-Mouse IgG magnetic beads (10 μL) were first washed twice in 50 mM PBS pH 7.4 by using an external magnet to allow easy separation. They were coated, then, with Primary Monoclonal Hepatitis A Virus Antibody (MAbI) in 50 mM carbonate buffer pH 9.6, shaking or 45 minutes at room temperature, separated with the magnet and washed again twice with Solution A. The coated beads were resuspended in 3% BSA in 50 mM carbonate buffer pH 9.6, shaking for 45 minutes at room temperature, washed again twice with Solution A and resuspended in Solution B. The beads were stored at +4 °C overnight, separated and washed twice with Solution A. HAV in 50 mM PBS pH 7.4 containing 0.1% BSA was added. After shaking for 45 minutes at room temperature, the beads were washed twice with Solution A. An alkaline phosphatase (AP) labeled anti-HAV monoclonal antibody (MAb-AP) in 50 mM PBS pH 7.4 was used to complete the immunological chain constituted by HAV and MAbI, immobilized directly on the beads, to reveal the sandwich complex format. The beads were shaken for 45 minutes at room temperature, washed twice with Solution A and resuspended in 50 mM PBS pH 7.4. After the immunochemical steps, the beads were localized onto the surface of SPEs with the aid of a magnet. The enzyme-substrate 1-naphthyl-phosphate (1-NPP) in 0.97 diethanolamine (DEA) buffer pH 9.8 containing 1 mM MgCl2 and 100 mM KCl was added to measure the electroactive product (Figure 1).

Figure 1. Protocol used in the ELIME detection.

2.3.2. Spiking of drinking water and water for vegetable processing

Drinking water was collected from the public water system of Rome, Italy. Vegetable processing water was collected from an industrial setting performing processing of ready-to-eat leafy vegetables and included water taken after its use for vegetable washing. One hundred mL of drinking water and vegetable processing water were spiked with different concentrations of HAV (strain HM175/18f) at concentrations ranging between 0 (L0) and ~10.000 g.c./mL (L6) including, for each level, intermediate dilutions (e.g. L5/6 = ~5.000 g.c./mL). The exact inoculum value (and expected value for quantification by RT-qPCR and ELIME assay) was confirmed through analysis of the highest concentration of the HAV suspension used for spiking.

2.3.3. Sample concentration for HAV detection

The concentration method for HAV was obtained by a modification of an organic solvent-based method taken from the literature [8,9] and the reference ISO method for HAV detection in food and water samples (ISO 15216-1:2017) [7]. Samples were filtered on the wrinkled side of the Zeta Plus™ 1MDS disc using an EZ-Stream® vacuum laboratory pump (Merck Millipore). Viruses adsorbed on the filter were eluted and recovered using 40 mL of Solution C, spinning at 35-40 rpm for 15 minutes. After adjusting pH to 7.0 ± 0.2 with HCl 5 N, the suspension was divided into two aliquots of equal volume, for ELIME assay detection, and real-time RT-qPCR analysis, respectively. The aliquot intended for RT-qPCR was concentrated according to ISO 15216-1:2017 [7], centrifuged at 4000 ×g for 15 minutes using Amicon Ultra centrifugal filter devices ultracel 100 KDa (Merck Millipore) and the viral concentrate was eluted with 0.5 mL of sterile PBS (Figure 2).

Figure 2. Extraction method for HAV from drinking water and water for vegetable processing.

2.3.4. RNA extraction and real-time RT-qPCR protocol

Nucleic acid extraction was performed using the NucliSens extraction kit and the MiniMag semi-automatic platform (bioMerieux, Paris, France) according to the manufacturer’s instructions. Eluted RNA (100 μL) was stored at −80 °C until molecular analysis. Real-time RT-qPCR was conducted using the amplification conditions, primers, probes, and reagents reported in the annexes of the ISO 15216 method [28,32,33]. Briefly, the assay was performed using the RNA UltraSense One-Step qRT-PCR System kit (Life Technologies) in a 25 μl reaction mixture containing 5 μl of RNA, 900 nM of reverse primer, 500 nM of forward primer, and 250 nM of the probe. The reactions were carried out in duplicate on a 7700 Sequence Detection System (Applied Biosystems) with the following amplification conditions: reverse transcription at 55 °C for 60 min, 95 °C for 5 min, and 45 cycles at 95 °C for 15 s, 60°C for 60 s, and 65°C for 60 s. For standard curve construction, tenfold dilutions of a quantified plasmid containing the target sequence were used (range 101 – 105 copies/l); quantifications were considered acceptable if standard curves displayed a slope between -3.1 and -3.6 and an R2 ≥ 0.98.

2.3.5. Calibration curve for immunosensor

The dose-response curve of the ELIME was constructed by fitting experimental data with a nonlinear four-parameter logistic calibration plot, as shown in the equation [30,34]:

y=y0+a1+xx0b 							Eq.1

in which a and y0 are the asymptotic maximum and minimum values, x0 is the IC50 and b is the slope. Each value of current I (μA) was normalized using the formula [30]:

I% = Ix-IminImax-Imin∙100 %  						Eq.2

with Imin the average value of the current at minimum concentration of HAV and Imax the average value of the current at maximum concentration of HAV.

The LOD, Eq. 3, and the limit of quantification (LOQ), Eq. 4, were calculated as follows [30,34]:

LOD = SB+3 SD 						Eq.3

LOQ = SB+10 SD 						Eq.4

where SB is the signal measured in the absence of HAV and SD is the standard deviation of the blank (10 replications).

3. Results and discussion

3.1. ELIME optimization

In this paper, a rapid system for HAV detection, based on ELIME assay, was evaluated. The system is based on the use of Goat Anti-Mouse IgG magnetic beads as a solid support for the immunochemical chain, using an array of 8 screen-printed electrodes (SPEs) as sensing platforms. An alkaline phosphatase-labelled anti-HAV monoclonal antibody (MAb-AP) was used as an electroactive reporter and to complete the immunological chain formed by HAV and primary monoclonal antibody against HAV (MAbI), immobilized directly on magnetic beads to reveal the sandwich complex. The magnetic beads were kept on SPEs with the support of a magnet and the enzyme-substrate 1-naphthyl-phosphate (1-NPP) was added to measure the electroactive product. For this goal, analytical parameters were optimized. To establish the best conditions for the sandwich ELIME, different dilutions of MAbI were tested using a fixed concentration of HAV (10-8 UI/mL), 3% Dry Milk (DM) as a blocking agent and MAb-AP diluted 1:10000 (v/v) in PBS with incubation times of 45 minutes (parameters from previous studies) [21] (Figure 3). The maximum electrochemical signals were observed at a concentration of 10 μg/mL MAbI, whereas no further current increase was observed at higher concentrations.

Figure 3. Effect of MAbI concentration on the signal. 3% Dry Milk (DM), [MAbI] 0, 5, 10, 15 μg/mL, [HAV] 10-8 UI/mL, [MAb-AP] 1:10000 (v/v) in PBS, 5 mg/mL 1-NPP; DPV measurement potential range 0-600 mV, the pulse width of 50 ms, pulse amplitude of 70 mV and a scan rate of 100 mV/s.

Different blocking agents were evaluated to reduce non-specific adsorption onto the surface of coated beads. 3% Bovine Serum Albumin (BSA), 3% Dry Milk (DM) and 1% Polyvinyl Alcohol (PVA) were tested by adding MAb-AP diluted 1:5000, 1:10000, 1:25000 (v/v) in PBS with no other bioreagent. The experimental results (Figure 4) revealed that BSA is the most effective blocking agent for MAb-AP diluted 1:25000 (v/v) in PBS from the stock solution.

Figure 4. Study of the blocking reagent at different concentrations of MAb-AP. 3% BSA, 3% DM and 1% PVA, [MAb-AP] 1:5000, 1:10000, 1:25000 (v/v) in PBS, 5 mg/mL 1-NPP; DPV measurement potential range 0-600 mV, pulse width of 50 ms, pulse amplitude of 70 mV and scan rate of 100 mV/s.

3.2. Analytical performances in buffer and real sample solution

After optimizing the antibody concentration and the blocking agent, a calibration curve was constructed using HAV strain HM175 (Figure 5). This ELIME assay achieves a detection limit (LOD) of 0.5 g.c./mL for the quantitative determination of HAV.

Figure 5. Calibration curve for HAV. [MAbI] 10 μg/mL, 3% BSA, [HAV] 0-10 g.c./mL, [MAb-AP] 1:25000 (v/v), 5 mg/mL 1-NPP; DPV measurement potential range 0-600 mV, the pulse width of 50 ms, pulse amplitude of 70 mV and a scan rate of 100 mV/s. Parameters: a=96.53, b=-3.03, x0=0.89,yo=-0.18. LOD=0.5 g.c./mL, RSD% 7%.

To investigate the applicability of the proposed methods for practical analysis, the immunosensor was used to evaluate the detection of different concentrations of HAV spiked in drinking water samples and in water used for ready-to-eat vegetable processing (washing). Results were compared with those obtained by the standard real-time RT-qPCR analysis (ISO 15216-1:2017) [7] which is typically used to estimate the presence of HAV in food and water samples. To compare the results of the two methods (ELIME and RT-qPCR), a correlation was established on the HM175/18f strain supplied by ISS between the units used in ELIME (IU/mL) and those used in RT-qPCR (g.c./mL), revealing that 1 UI/mL=2×106 g.c./mL with R2=0.997.

In experimentally contaminated water samples, the ELIME assay detected HAV at the lowest spiking level (L2) in both types of water samples (Tables 1-2), demonstrating a greater sensitivity than the ISO 15216-1 method and validating a detection limit of 0.5 g.c./mL (roughly equivalent to 0.5 viruses/mL, given the correspondence between a genome and a viral particle). The linearity of results was modest, particularly in vegetable washing water, where organic and disinfectant residues reduced the efficiency of both ELIME and PCR analysis. However, the underestimation of viral concentration in the spiked samples did not negatively affect the overall ELIME sensitivity.

Table 1. Assessment of the ELIME essay on drinking water.

| Spiking Level | HAV expected value (g.c./mL) † | HAV measured value |  |
| --- | --- | --- | --- |
|  |  | ELIME assay (g.c./mL) | RT-qPCR (g.c./mL) |
| L0 | - | - | - |
| L2 | 0.8 ± 0.12 | 0.92 | - |
| L2/3 | 4.1 ± 0.6 | 1.15 | 0.56* ± 0.3 |
| L3 | 8 ± 1 | 1.45 | 0.56* ± 0.2 |
| L3/4 | 41 ± 6 | 2.59 | 2.70 ± 0.8 |
| L4 | 82 ± 12 | 3.24 | 4.22 ± 0.7 |
| L4/5 | 411 ± 60 | 4.42 | 35.44 ± 3 |
| L5 | 822 ± 121 | 103.54 | 119 ± 9 |
| L5/6 | 4108 ± 603 | 531.06 | 952 ± 65 |
| L6 | 8216 ± 1206 | 1088.84 | 2684 ± 154 |

† Based on quantification of HAV stock used for spiking of samples

*Value below the analytical LOQ of the real-time RT-qPCR

Table 2. Assessment of the ELIME assay on vegetables rinsing water.

| Spiking Level | HAV expected value (g.c./mL) † | HAV measured value |  |
| --- | --- | --- | --- |
|  |  | ELIME assay (g.c./mL) | RT-qPCR (g.c./mL) |
| L0 | - | - | - |
| L2 | 0.91 ± 0.34 | 1.50 | - |
| L2/3 | 4.56 ± 1.67 | 1.66 | 0.19* ± 0.16 |
| L3 | 9.12 ± 3.34 | 1.69 | 0.40* ± 0.09 |
| L3/4 | 45.59 ± 16.67 | 1.98 | 0.53* ± 0.36 |
| L4 | 91.18 ± 33.34 | 2.18 | 0.96* ± 0.73 |
| L4/5 | 455.89 ± 166.68 | 3.60 | 4.04 ± 2.01 |
| L5 | 911.78 ± 333.36 | 30.83 | 6.46 ± 0.30 |
| L5/6 | 4558.94 ± 1666.79 | 135.71 | 86.85 ± 5.21 |
| L6 | 9117.87 ± 3333.58 | 229.09 | 93.02 ± 3.53 |

† Based on quantification of HAV stock used for spiking of samples

*Value below the analytical LOQ of the real-time RT-qPCR

3.3. Comparison of the performance and cost-benefit analysis

The performance of the ELIME sensor presented in this study was benchmarked against previously reported HAV sensors, as detailed in Table 3. The analytical capabilities of the ELIME method are noteworthy, particularly given its simplicity of assembly and a projected cost that is approximately five times lower than existing alternatives. Furthermore, it is critical to highlight that the ELIME assay’s features address significant challenges in the microbiological monitoring of water sources used in food production. Its application facilitates on-site monitoring along production lines, significantly reduces costs, and thereby promotes more comprehensive and frequent control measures. Consequently, the ELIME method represents a substantial advancement in the detection of HAV in complex real-world samples.

Table 3. The performance of the ELIME sensor and previously reported HAV sensors.

| Transductor | LOD | Sample | Reference |
| --- | --- | --- | --- |
| MNPs–pDA ELIME | 10-11 IU/mL | Tap water | [21] |
| MIP-SPE | 4.6×10−4 ng/mL | Human plasma | [35] |
| DNA-SPE | 6.94 fg/µL | PBS | [36] |
| ELIME | 0.5 g.c./mL | Drinking and vegetables rinsing water | This work |

4. Conclusions

Currently, spectrophotometric ELISA and qPCR are the primary methods for diagnosing HAV infections in humans and monitoring the virus in food, respectively. Notably, serological testing for HAV, commonly used to investigate abnormal liver function, may face challenges in accurately diagnosing acute infection, particularly when IgG testing is inconclusive or low-level anti-HAV IgM results are observed.

In this study, we present a robust analytical method as an alternative to real-time RT-qPCR for detecting HAV in water samples. The developed ELIME assay effectively identified HAV in both drinking water and vegetable rinsing water, demonstrating high comparability to conventional reference methods. These findings highlight its potential application in agricultural contexts, such as the monitoring of irrigation water, and in food processing environments for real-time checks in washing systems, where viruses detached from treated vegetables may remain suspended and pose a risk of cross-contamination to subsequent batches. Furthermore, due to its straightforward design, this method can be adapted for the detection of a wide range of environmentally significant pathogens by simply substituting the analyte-specific bioreceptor.

Author Contributions: Formal Analysis, Investigation, Data Curation, Writing – Original Draft Preparation C.A.; Data Curation, Writing – Review & Editing R.C., Visualisation A.C.; Conceptualization, Resources, Visualisation D.M.; Visualization, Resources  L.C.; Visualization, Resources G. LR; Conceptualization, Methodology, Validation, Formal Analysis, Resources, Data Curation, Writing – Review & Editing, Supervision, Project Administration E.S.; Conceptualization, Methodology, Valisation, Formal Analysis, Resources, Data Curation, Writing – Review & Editing, Supervision, Project Administration L.M.

References

[1]	P. Van Damme, R.M. Pintó, Z. Feng, F. Cui, A. Gentile, D. Shouval, Hepatitis A virus infection, Nat Rev Dis Primers 9 (2023) 51. https://doi.org/10.1038/s41572-023-00461-2.

[2]	X. Wang, J. Ren, Q. Gao, Z. Hu, Y. Sun, X. Li, D.J. Rowlands, W. Yin, J. Wang, D.I. Stuart, Z. Rao, E.E. Fry, Hepatitis A virus and the origins of picornaviruses, Nature 517 (2015) 85–88. https://doi.org/10.1038/nature13806.

[3]	A. Fallucca, V. Restivo, M.C. Sgariglia, M. Roveta, C. Trucchi, Hepatitis a Vaccine as Opportunity of Primary Prevention for Food Handlers: A Narrative Review, Vaccines 11 (2023) 1271. https://doi.org/10.3390/vaccines11071271.

[4]	M.T.M. Shata, H.F. Hetta, Y. Sharma, K.E. Sherman, Viral hepatitis in pregnancy, Journal of Viral Hepatitis 29 (2022) 844–861. https://doi.org/10.1111/jvh.13725.

[5]	G.R. Takuissu, S. Kenmoe, J.T. Ebogo-Belobo, C. Kengne-Ndé, D.S. Mbaga, A. Bowo-Ngandji, J.L. Ndzie Ondigui, R. Kenfack-Momo, S. Tchatchouang, J. Kenfack-Zanguim, R. Lontuo Fogang, E. Zeuko’o Menkem, G.I. Kame-Ngasse, J.N. Magoudjou-Pekam, C. Veneri, P. Mancini, G. Bonanno Ferraro, M. Iaconelli, L. Orlandi, C. Del Giudice, E. Suffredini, G. La Rosa, Occurrence of Hepatitis A Virus in Water Matrices: A Systematic Review and Meta-Analysis, IJERPH 20 (2023) 1054. https://doi.org/10.3390/ijerph20021054.

[6]	A. Tiwari, W. Ahmed, S. Oikarinen, S.P. Sherchan, A. Heikinheimo, G. Jiang, S.L. Simpson, J. Greaves, A. Bivins, Application of digital PCR for public health-related water quality monitoring, Science of The Total Environment 837 (2022) 155663. https://doi.org/10.1016/j.scitotenv.2022.155663.

[7]	L. Pérez-Lavalle, E. Carrasco, A. Valero Diaz, Microbiological criteria: Principles for their establishment and application in food quality and safety, Ital J Food Safety 9 (2020). https://doi.org/10.4081/ijfs.2020.8543.

[8]	S.G. Goh, N. Saeidi, X. Gu, G.G.R. Vergara, L. Liang, H. Fang, M. Kitajima, A. Kushmaro, K.Y.-H. Gin, Occurrence of microbial indicators, pathogenic bacteria and viruses in tropical surface waters subject to contrasting land use, Water Research 150 (2019) 200–215. https://doi.org/10.1016/j.watres.2018.11.058.

[9]	E.C. Hounkpe, P. Sessou, S. Farougou, I. Dotche, G. Daube, V. Delcenserie, P. Azokpota, N. Korsak, Hygiene practices of food of animal origin operators in primary schools in the Mono Department of Benin. A cross-sectional study, Heliyon 9 (2023) e17135. https://doi.org/10.1016/j.heliyon.2023.e17135.

[10]	C.P. Gerba, Environmentally Transmitted Pathogens, in: Environmental Microbiology, Elsevier, 2015: pp. 509–550. https://doi.org/10.1016/B978-0-12-394626-3.00022-3.

[11]	O. Gholizadeh, S. Akbarzadeh, M. Ghazanfari Hashemi, M. Gholami, P. Amini, Z. Yekanipour, R. Tabatabaie, S. Yasamineh, P. Hosseini, V. Poortahmasebi, Hepatitis A: Viral Structure, Classification, Life Cycle, Clinical Symptoms, Diagnosis Error, and Vaccination, Canadian Journal of Infectious Diseases and Medical Microbiology 2023 (2023) 1–17. https://doi.org/10.1155/2023/4263309.

[12]	G.R. Takuissu, S. Kenmoe, J.T. Ebogo-Belobo, C. Kengne-Ndé, D.S. Mbaga, A. Bowo-Ngandji, J.L. Ndzie Ondigui, R. Kenfack-Momo, S. Tchatchouang, J. Kenfack-Zanguim, R. Lontuo Fogang, E. Zeuko’o Menkem, G.I. Kame-Ngasse, J.N. Magoudjou-Pekam, C. Veneri, P. Mancini, G. Bonanno Ferraro, M. Iaconelli, L. Orlandi, C. Del Giudice, E. Suffredini, G. La Rosa, Occurrence of Hepatitis A Virus in Water Matrices: A Systematic Review and Meta-Analysis, IJERPH 20 (2023) 1054. https://doi.org/10.3390/ijerph20021054.

[13]	E. Cuevas-Ferrando, A. Martínez-Murcia, A. Pérez-Cataluña, G. Sánchez, W. Randazzo, Assessment of ISO Method 15216 to Quantify Hepatitis E Virus in Bottled Water, Microorganisms 8 (2020) 730. https://doi.org/10.3390/microorganisms8050730.

[14]	D. Wang, J. Cao, Z. Tian, B. Fang, X. Qi, Z. Lei, L. Liu, J. Zhu, L. Ma, Development of a new concentration method for Hepatitis A virus detection (ISO 15216–2:2019) in Manila clams (Ruditapes philippinarum), LWT 172 (2022) 114172. https://doi.org/10.1016/j.lwt.2022.114172.

[15]	L. Fabiani, E. Pucci, E. Delibato, G. Volpe, S. Piermarini, D. De Medici, F. Capuano, G. Palleschi, ELIME assay vs Real-Time PCR and conventional culture method for an effective detection of Salmonella in fresh leafy green vegetables, Talanta 166 (2017) 321–327. https://doi.org/10.1016/j.talanta.2017.01.071.

[16]	A. Tiwari, D.M. Oliver, A. Bivins, S.P. Sherchan, T. Pitkänen, Bathing Water Quality Monitoring Practices in Europe and the United States, IJERPH 18 (2021) 5513. https://doi.org/10.3390/ijerph18115513.

[17]	F. Watzinger, K. Ebner, T. Lion, Detection and monitoring of virus infections by real-time PCR, Molecular Aspects of Medicine 27 (2006) 254–298. https://doi.org/10.1016/j.mam.2005.12.001.

[18]	J. Li, W. Lin, P. Du, W. Liu, X. Liu, C. Yang, R. Jia, Y. Wang, Y. Chen, L. Jia, L. Han, W. Tan, N. Liu, J. Du, Y. Ke, C. Wang, Comparison of reverse-transcription qPCR and droplet digital PCR for the detection of SARS-CoV-2 in clinical specimens of hospitalized patients, Diagnostic Microbiology and Infectious Disease 103 (2022) 115677. https://doi.org/10.1016/j.diagmicrobio.2022.115677.

[19]	W. Randazzo, A. Vasquez-García, R. Aznar, G. Sánchez, Viability RT-qPCR to Distinguish Between HEV and HAV With Intact and Altered Capsids, Front. Microbiol. 9 (2018) 1973. https://doi.org/10.3389/fmicb.2018.01973.

[20]	S. Sakamoto, W. Putalun, S. Vimolmangkang, W. Phoolcharoen, Y. Shoyama, H. Tanaka, S. Morimoto, Enzyme-linked immunosorbent assay for the quantitative/qualitative analysis of plant secondary metabolites, J Nat Med 72 (2018) 32–42. https://doi.org/10.1007/s11418-017-1144-z.

[21]	P.V. Hornbeck, Enzyme‐Linked Immunosorbent Assays, CP in Immunology 110 (2015). https://doi.org/10.1002/0471142735.im0201s110.

[22]	A.J. Van Hengel, E. Anklam, S.L. Taylor, S.L. Hefle, Analysis of food allergens. Practical applications, in: Food Toxicants Analysis, Elsevier, 2007: pp. 189–229. https://doi.org/10.1016/B978-044452843-8/50008-0.

[23]	S.W. Mes, D.A.M. Heideman, E. Bloemena, A. Brink, M. Bogaarts, C.R. Leemans, R.H. Brakenhoff, Development and Validation of a Novel and Rapid Molecular Detection Method for High-Risk Human Papillomavirus in Formalin-Fixed, Paraffin-Embedded Tumor Tissue, The Journal of Molecular Diagnostics 22 (2020) 262–271. https://doi.org/10.1016/j.jmoldx.2019.10.009.

[24]	K.L. Singampalli, J. Li, P.B. Lillehoj, Rapid magneto-enzyme-linked immunosorbent assay for ultrasensitive protein detection, Analytica Chimica Acta 1225 (2022) 340246. https://doi.org/10.1016/j.aca.2022.340246.

[25]	E. Delibato, G. Volpe, D. Romanazzo, D. De Medici, L. Toti, D. Moscone, G. Palleschi, Development and Application of an Electrochemical Plate Coupled with Immunomagnetic Beads (ELIME) Array for Salmonella enterica Detection in Meat Samples, J. Agric. Food Chem. 57 (2009) 7200–7204. https://doi.org/10.1021/jf901181m.

[26]	A. Corradini, M. Cecchini, M. Trevisani, A Rapid Enzyme-Linked Immunomagnetic Electrochemical (ELIME) Assay for the Detection of Escherichia coli O26 in Raw Milk, Food Anal. Methods 13 (2020) 1366–1370. https://doi.org/10.1007/s12161-020-01758-8.

[27]	L. Fabiani, E. Delibato, G. Volpe, S. Piermarini, D. De Medici, G. Palleschi, Development of a sandwich ELIME assay exploiting different antibody combinations as sensing strategy for an early detection of Campylobacter, Sensors and Actuators B: Chemical 290 (2019) 318–325. https://doi.org/10.1016/j.snb.2019.03.140.

[28]	L. Micheli, A. Fasoli, A. Attar, D.T. Donia, M. Divizia, A. Amine, G. Palleschi, P.A. Salazar Carballo, D. Moscone, An ELIME assay for hepatitis A virus detection, Talanta 234 (2021) 122672. https://doi.org/10.1016/j.talanta.2021.122672.

[29]	Y. Masago, Y. Konta, S. Kazama, M. Inaba, T. Imagawa, K. Tohma, M. Saito, A. Suzuki, H. Oshitani, T. Omura, Comparative Evaluation of Real-Time PCR Methods for Human Noroviruses in Wastewater and Human Stool, PLoS ONE 11 (2016) e0160825. https://doi.org/10.1371/journal.pone.0160825.

[30]	R. Cancelliere, A. Di Tinno, A. Cataldo, S. Bellucci, S. Kumbhat, L. Micheli, Nafion-based label-free immunosensor as a reliable warning system: The case of AFB1 detection in cattle feed, Microchemical Journal 191 (2023) 108868. https://doi.org/10.1016/j.microc.2023.108868.

[31]	L. Celio, M. Ottaviani, R. Cancelliere, A. Di Tinno, P. Panjan, A.M. Sesay, L. Micheli, Microfluidic Flow Injection Immunoassay System for Algal Toxins Determination: A Case of Study, Front. Chem. 9 (2021) 626630. https://doi.org/10.3389/fchem.2021.626630.

[32]	M.G. Hofmeister, M.A. Foster, E.H. Teshale, Epidemiology and Transmission of Hepatitis A Virus and Hepatitis E Virus Infections in the United States, Cold Spring Harb Perspect Med 9 (2019) a033431. https://doi.org/10.1101/cshperspect.a033431.

[33]	R. Wu, B. Meng, M. Corredig, M.W. Griffiths, Rapid Detection of Hepatitis A Virus in Foods Using a Bioluminescent Assay in Real-Time (BART) and Reverse Transcription Loop-Mediated Isothermal Amplification (RT-LAMP) Technology, Food Environ Virol 15 (2023) 144–157. https://doi.org/10.1007/s12560-022-09548-7.

[34]	R. Cancelliere, T. Cosio, E. Campione, M. Corvino, M.P. D’Amico, L. Micheli, E. Signori, G. Contini, Label-free electrochemical immunosensor as a reliable point-of-care device for the detection of Interleukin-6 in serum samples from patients with psoriasis, Front. Chem. 11 (2023) 1251360. https://doi.org/10.3389/fchem.2023.1251360.

[35]	M. Antipchik, J. Reut, A.G. Ayankojo, A. Öpik, V. Syritski, MIP-based electrochemical sensor for direct detection of hepatitis C virus via E2 envelope protein, Talanta 250 (2022) 123737. https://doi.org/10.1016/j.talanta.2022.123737.

[36]	M. Manzano, S. Viezzi, S. Mazerat, R.S. Marks, J. Vidic, Rapid and label-free electrochemical DNA biosensor for detecting hepatitis A virus, Biosensors and Bioelectronics 100 (2018) 89–95. https://doi.org/10.1016/j.bios.2017.08.043.